## 2019年大学生竞赛越野车——总布置与车架设计

摘 要

在参考巴哈大赛竞赛规则和本校往届参赛赛车的数据下，本文对Baja赛车进行了总布置及车架的设计进行介绍。首先介绍如何制定出合适的总布置参数，并对各大系统进行初步布置，然后汇集所有的系统进行总装配，得到最终的整车参数。在进行赛车车架的设计时，根据实际情况来对车架进行选型，规划车架设计指标。本次设计对几种轻质材料和大赛推荐的材料做了调查计算，并选取了合适的规格。然后在使用UG软件来对Baja赛车车架进行了各个结构件的三维模型设计，并且使用软件提供的人体模型来检验车架是否符合人机工程学。使用Hyper mesh软件对Baja赛车车架进行有限元网格的划分，并且分析和检验了车架在多个不同工况下变形量与受力是否符合使用要求，以及对车架质量进行优化。并且设计了具有相同结构并使用铝合金材料的车架来进行对比，观察并比较两者的优劣，作为对轻型材料车架的探索，最后在对车架进行模态分析，检验车架是否会在正常行驶中发生共振。最后再介绍了车架及附属部件的加工制造方法。

关键词：巴哈大赛   车架   铝合金   Hyper mesh   总布置   质量优化

ABSTRACT

This paper introduces the general arrangement and frame design of Baja racing car by referring to the competition rules of Baja racing and the data of previous racing cars of our university. Firstly, it introduces how to work out the appropriate general layout parameters, and preliminary layout of the major systems. Finally, it collects all the systems for general assembly, and obtains the final vehicle parameters. In the design of racing car frame, according to the actual situation to select the type of frame, planning frame design indicators. In this design, several lightweight materials and materials recommended by the competition were investigated and calculated, and appropriate specifications were selected. Then the three-dimensional design of Baja racing frame is carried out by UG software, and the human body model provided by the software is used to check whether the frame conforms to ergonomics. The Hyper Mesh software is used to divide the finite element mesh of the racing car frame, and the deformation and stress of the frame under different working conditions are analyzed and checked, and the quality of the frame is optimized. And the frame with the same structure and using alloy aluminium material is designed for comparison. Observing and comparing the advantages and disadvantages of the two is regarded as the exploration of light material frame. Finally, the modal analysis of the frame is carried out to check whether the frame will resonate in normal driving. Finally, the manufacturing methods of frame and accessory parts are introduced.

Key words: Baja competition    Frame    Aluminum Alloy    Hyper mesh

General arrangement     Quality optimization

以上内容仅为本文档的试下载部分，为可阅读页数的一半内容。如要下载或阅读全文，请访问：https://d.book118.com/016025025121010032