小型增程机械臂直流电机驱动电子控制器的软件设计

The software design of the electronic controller driven by the dc motor of the miniature extended-range manipulatoron

# 摘  要

自从工业化及科技革命以来，越来越多的工程应用中一台直流电机无法提供足够的动力和操作，因此在工程应用中需要多台电机协作，在传统的工程实践中，主要通过机械连接实现直流电机的转矩、转速、转角和增程控制来实现动作跟随操控，但是由于其输出参数有限并且很多情况下不适宜用机械连接，因此直流电动机跟随控制是一个十分有应用价值的研究方向。

本文以多直流电机的转矩、转速、转角和增程控制为研究对象，建立控制单元的数学模型，目标在于建立多直流电机转矩、转速、转角和增程控制单元。本控制单元，主要通过主从控制，采用PID控制器对控制单元实行控制与优化，对于直流电机的转矩、转速、转角和增程测量采用霍尔开关传感器测量，由于测量过程中存在有不可消除的噪声，严重时会使控制单元失控，因此通过加入卡尔曼滤波器实现对测量输出的滤波，使控制单元的工作更加稳定。实际测试表明在PID控制与卡尔曼滤波的联合作用下，控制系统鲁棒性好，响应快速，能够满足实际需求。之后，又在程序中更进一步实现了模糊PID控制器，以提高系统的抗干扰性能。

在此基础上，我们还搭建了实物来验证数学模型的真实性能。在搭建的实物中由STC12C56AD作为MCU控制4个370减速电机，结合相应程序在实验中取得了良好的成果。

关键词 PID控制；卡尔曼滤波；MCU

# Abstract

Since the industrialization and the revolution of science and technology, more and more application of a dc motor can't provide enough power and operation, so need more motor coordination in the engineering application, in the practice of the traditional engineering, mainly by mechanical connection of dc motor torque, speed, Angle and the extended range control to implement the action follow control, but due to its limited output parameters and in many cases not appropriate to use mechanical connection, dc motor tracking control, therefore, is a research direction of application value.

This paper takes the torque, speed, Angle and extended range control of the multi-dc motor as the research object, establishes the mathematical model of the control unit, and aims to establish the torque, speed, Angle and extended program control unit of the multi-dc motor.This control unit, mainly through the master-slave control, the PID controller to the control unit controls and optimization, for dc motor torque, speed, Angle and the extended range measurement using hall switch sensor, because there is not to eliminate the noise of the measurement process, serious when they make the control unit is out of control, so by joining the kalman filter for the measurement of the output filter, the work of the control unit is more stable.The actual test shows that under the combined action of PID control and kalman filter, the control system has good robustness, fast response and can meet the actual needs.Then,

以上内容仅为本文档的试下载部分，为可阅读页数的一半内容。如要下载或阅读全文，请访问：https://d.book118.com/137152020161006046