# 摘  要

本次设计选取珠海市某办公楼建筑为设计对象，该办公楼共六层，该设计目的是设计一套合理的中央空调系统为工作环境中的办公人员提供一个健康、安全、舒适的室内环境。

在本次工程设计中，通过查找资料总结同类型建筑工程经验，总结办公楼建筑的负荷特征与空调方案等。这座办公楼占地面积约有一万多平米，通过使用冷负荷系数法对冷负荷进行计算，经过计算得到总的冷负荷有1336kW，然后根据计算得到的冷负荷选取两种不同冷源形式进行对比，通过经济性分析比较不同冷源方案的初投资和年运行费用，最终确定最佳方案为螺杆式冷水机组。

根据以往的建筑经验，在空调设计中，一般空调房间面积较小，人员活动时间固定，通常使用的是空气-水系统，典型代表是风机盘管加新风系统，末端装置为风机盘管，通过对每个房间进行风量计算，选出型号合适风机盘管。然后根据假定流速法进行风管水力计算，根据最后的计算结果进行水力平衡。水系统的由于制冷机房设在一楼，空调水系统采用下供下回形式。由于只设计夏季供冷，因此管路选用两管制。最后对水力计算得出的结果进行阻力平衡。制冷机房则是根据和设计规范进行布置。

本次设计以节能与舒适为核心目标，运用自己掌握的建筑实际情况，查阅相关资料，保证空调房间可以有一个良好的空调室内环境。

关键词: 中央空调、风机盘管、冷水机组、节能与舒适

# Abstract

This design selects an office building in Zhuhai as the design object. The office building has six floors. The design purpose is to design a reasonable central air conditioning system to provide a healthy, safe and comfortable indoor environment for office workers in the working environment. .

In this project design, through the search for data to summarize the same type of construction engineering experience, summarize the load characteristics of the office building and air conditioning solutions. The office building covers an area of ​​more than 10,000 square meters. The cooling load is calculated by using the cold load coefficient method. After calculation, the total cooling load is 1336 kW, and then two different cold source forms are selected according to the calculated cooling load. For comparison, the initial investment and annual operating costs of different cold source schemes were compared through economic analysis, and the best solution was finally determined to be a screw chiller.

According to the previous building experience, in the air-conditioning design, the general air-conditioned room area is small, the personnel activity time is fixed, usually the air-water system is used, the typical representative is the fan coil plus fresh air system, and the end device is the fan coil, through Calculate the air volume for each room and select the appropriate fan coil for the model. Then, the duct hydraulic calculation is performed according to the assumed flow rate method, and the hydraulic balance is performed based on the final calculation result. The water system is located on the first floor of the refrigerating machine room, and the air conditioning water system is in the form of a lower supply and a lower. Since only the summer cooling is designed, the pipeline is controlled by two. Finally, the resistance calculation is performed on the results of the hydraulic calculation. The refrigerating machine room is arranged according to the design specifications.

The design aims to save energy and comfort, use the actual situation of the building, and check the relevant information to ensure that the air-conditioned room can have a good air-conditioned indoor environment.

Keywords: Central air conditioning; Fan coil; Chiller, Energy saving and comfortable.

以上内容仅为本文档的试下载部分，为可阅读页数的一半内容。如要下载或阅读全文，请访问：https://d.book118.com/318075140124006044