- 原文

A Design of Remote Monitoring System based on 3G and Internet Technology

Shouxian WEN

Key Lab of Intelligent Computing and Signal Processing of

Ministry of Education, Anhui University

Hefei, China

Xingguo SUN

Key Lab of Intelligent Computing and Signal Processing of

Ministry of Education, Anhui University

Hefei, China

Lei XU

Key Lab of Intelligent Computing and Signal Processing of

Ministry of Education, Anhui University

Hefei, China

Xiaohui LI

Key Lab of Intelligent Computing and Signal Processing of

Ministry of Education, Anhui University

Hefei, China

Abstract — Aiming at the low-rate and short-distance

communication in existing temperature and humidity monitoring

system, this paper designs a remote monitoring system based on

3G and Internet technology. This system consists of data

acquisition module and data remote transmission module.

Atmega128 is used to be as a master control chip of data

acquisition module, which realizes the data acquisition and

pretreatment, and the collected data is transmitted to the host

node by the low-power wireless transmission module NRF905

which is installed on slave nodes, and then the converged data is

transmitted to Internet by 3G module. The system can realize

remote monitoring, centralized management and so on. The

experimental results show that the system has the advantages

of

low installation cost, low consumption, high sensitivity, high

accuracy, long-distance communication and high rate etc.

Keywords-DS18B20; 3G; NRF905; Standard Definition (SD)

card; Data acquisition subsystem; Data remote transmission

subsystem.

I. INTRODUCTION

With the rapid development of computer networks and

communication technologies, remote monitoring technology in

the data acquisition and analysis has been widely used [1-7].

The traditional data acquisition system generally uses the

computer (PC) – slave node mode. Slave nodes use a wired

connection so that wiring tedious, poor scalability. Especially

for the vast space environment, if a wired connection is used,

the cost and power consumption is high. In addition, in the

traditional wired data transmission system, RS-485 protocol

which transmission distance is short is frequently used in the

communication between PC and slave nodes. The existing

Zigbee protocol can set up remote communication

transmission platform through the wireless AD hoc network,

but its transmission rate is low. NRF905 wireless module

transmission rate is high, but it lacks network protocol [8-

11].The data remote transmission system which composes of

3G and Internet can achieve a wide range of central control

and remote operation. However, if 3G module is used in each

slave node, the cost will be unacceptable

For these reasons, this paper designs a remote monitoring

system based on 3G and Internet technology. The system

consists of data acquisition and data remote transmission

subsystem. First, the data acquisition subsystem acquires

information, such as temperature, humidity, and then uploads

the data to the server that has a fixed IP add ress through

remote data transmission subsystem. Through the reasonable

design of the application software, the global authorized user

can realize the remote data of observation.

II. SYSTEM FRAMEWORK

The system framework is shown in Figure 1. The system

framework consists of data acquisition subsystem and data

remote transmission subsystem. Data acquisition subsystem

consists of one host node and several slave nodes. The host and

each slave node exchange data by using NRF905 wireless

module. The slave node collects information by digital sensors

which are inside of the node, and sends it to the host node in

form of packets. The converged data via 3G is connected to

Internet, and uploaded to the server. The server is responsible

for forwarding the data to the client. Monitoring results will be

shown to the users after the client data analysis.

Figure 1. The system framework

Figure 1. The system framework

III. THEDESIGN OFDATA ACQUISITION SUBSYSTEM

A. The design of hardware circuit

1) The design of host node hardware circuit

As a coordinator for the entire network, the host node is

mainly responsible for sending network signal, establishing the

network, managing the network node, storing network node

information, and finishing the transmission of information.

以上内容仅为本文档的试下载部分，为可阅读页数的一半内容。如要下载或阅读全文，请访问：https://d.book118.com/955312142020011120